My Home Assistant Notifier

Imagine having a versatile smart home notifier that seamlessly integrates into your setup, providing sound and speech notifications, two-way voice communication with your Home Assistant, and even the ability to enjoy music and radio—all without the need for additional Bluetooth speakers. Whether you’re automating daily tasks or creating the perfect ambiance, a notifier like this can elevate your smart home experience.

What’s more, this device can be easily connected to existing built-in ceiling speakers, making it an unobtrusive yet powerful addition to any room. It simplifies smart home audio integration while delivering functionality and flexibility for a wide range of uses.

With these goals in mind, I set out to create my own custom notifier. After two previous attempts, which taught me a lot about what worked and what didn’t, I’m excited to share the third and most refined iteration of my Home Assistant notifier. Here’s how I brought it to life.

This is the third iteration of my speaker system for my Home Assistant smart home application. The first two versions were overly complicated and challenging to manage. They incorporated active amplifiers, which not only proved unsuitable for my speakers but also introduced several issues. These included significant interference, the requirement for high-voltage power to operate, and excessive heat generation that compromised my PLA-printed cases.

For the third iteration, I decided to simplify the design and focus on creating a practical, low-maintenance solution. I opted for a basic D-Class low-voltage amplifier to hook up to the speaker only when needed. This design can also run on battery power, making it much more versatile and easier to implement.

The result is a streamlined notifier with just three small, key components:

A low-power D-Class amplifier – Efficient and capable of driving the speaker without overheating.

A rechargeable battery module – To enable standalone operation without the need for constant external power.

A compact speaker unit – Chosen for compatibility and to ensure clear, interference-free audio output.

This iteration prioritizes simplicity and reliability over complexity. It is less prone to overheating and interference, making it ideal for integrating into my smart home setup. I’m thrilled with how this version turned out and look forward to testing its performance further!

PCB Modules

For this iteration of my Home Assistant notifier, I used a combination of compact and efficient PCB modules to simplify the design and improve functionality:

Lolin32 Mini Board (or Clones)
This board was an excellent choice due to its integrated Li-Po battery terminal, charger, and protection circuits. These features eliminated the need for additional modules to manage the battery. However, it lacked a voltage divider to measure the battery's voltage—a critical feature for battery-powered devices. To address this, I soldered a voltage divider circuit between an available GPIO pin, ground, and the positive terminal of the battery.

UDA1334 Stereo Driver for Audio Output
Initially, I considered using a MONO MAX98357 amplifier board that operates on the I2S protocol, but I didn’t have one on hand. Instead, I opted for the UDA1334 stereo driver, which lacks built-in amplifiers. This turned out to be an advantage for my setup since it allowed me to pair it with a separate amplifier module, giving me more control over the amplification process.

PAM8403 D-Class Amplifier Board
For amplification, I used a compact PAM8403 D-Class amplifier board. While it’s not the most ideal choice due to its limitations in power and sound quality, it was readily available and sufficient for the needs of this project. Despite its simplicity, it integrates well into the overall design, providing enough output to drive the connected speaker.

Battery and Case Design

For the case design, I utilized existing 3D models downloaded for free from grabcad.com—huge thanks to their creators for sharing such useful resources!

When I couldn’t find a suitable model, I turned to CAD software to create one from scratch. This process involved using PCB photos and a caliper for precise measurements. While it’s not overly complicated, it does take time and effort to ensure exact dimensions. I had to carefully measure, cross-check, and verify sizes against known component dimensions, like header pin spacing and other reference points, to ensure everything fit perfectly.

For the power source, I picked up an 1800mAh Li-Po module from a local store. Conveniently, it came with a pre-crimped connector compatible with the Lolin32 Mini board, which made the assembly process much easier and saved me time.

Next, I began designing the case for my future device. If you’re still with me and not bored by now, I’d be happy to walk you through the process. Of course, feel free to skip this part if case design isn't your thing.

Once I finalized the design, I sent it to my recently purchased Anycubic Cobra 3 Combo 3D printer. This amazing printer handled the task beautifully, and I was impressed with the print quality and precision it delivered.

After printing the case, I carefully laid out all the components inside, securing them in place using double-sided silicon adhesive tape. This method ensured a clean and stable assembly without the need for screws or complex mounts. Finally, I soldered all the connections together, bringing the entire project to life.

Testing and Sound Quality

To test the device, I connected it to a passive ceiling-mounted speaker I had on hand. While the sound quality from this basic speaker wasn’t particularly impressive, it was more than sufficient for its primary purposes—playing soft background music and, more importantly, delivering sound and speech notifications from Home Assistant.

For a more thorough test, I also hooked up the notifier to my Edifier R1380DB active speakers, and the results were impressive. The UDA1334-based I2S driver delivered surprisingly high-quality sound given its compact size and affordable price. The clarity and fidelity of the audio exceeded my expectations, making it a great fit for use with higher-quality speakers.

Wrapping Up

That’s all I’ve got for now! If you enjoyed this project and want to see more, please don’t forget to like this video and subscribe to my channel for updates on future projects. I’d love to hear your thoughts, so feel free to leave comments below—your feedback and ideas are always welcome!

As always, you can find the list of parts used, schematics, and 3D models for printing in the description of this video.

Thanks for watching, and have fun with your own builds! See you next time!

In this video, I showcase the third iteration of my Home Assistant notifier—a simplified and practical DIY project for smart home enthusiasts. Unlike the previous versions, this design focuses on ease of use, reliability, and efficient performance.

Highlights of the project include:

- A compact design powered by the Lolin32 Mini board with Li-Po battery support.

- A UDA1334 stereo driver for high-quality audio output and a PAM8403 D-Class amplifier for sound amplification.

- Custom-designed and 3D-printed casing using the Anycubic Cobra 3 Combo printer.

- Testing with both a passive ceiling-mounted speaker and Edifier R1380DB active speakers, showcasing versatile sound capabilities.

Whether you're looking to create your own notifier for sound and speech notifications or just enjoy tinkering with smart home projects, this video provides a detailed walkthrough.

🔧 Resources:

- Parts List: Lolin32 mini, UDA1334 I2S stereo module, Capacitor 400-3000mF, some wires, PAM8403 D-Class amplifier module.

- Schematics:

https://community.home-assistant.io/t/i2s-stereo-to-play-mp3-tts-from-flash-on-boot/594740/6?u=elik745i

https://community.home-assistant.io/t/turn-an-esp8266-wemosd1mini-into-an-audio-notifier-for-home-assistant-play-mp3-tts-rttl/211499/224?u=elik745i

- 3D Models for Printing: https://www.thingiverse.com/thing:6910612

Find all these resources in the video description!

Don’t forget to like, comment, and subscribe for more exciting DIY projects. Share your thoughts or questions in the comments section below—I’d love to hear from you. Thanks for watching, and have fun creating your own projects!

YouTube Tags:

Home Assistant, DIY notifier, smart home projects, Lolin32 Mini, UDA1334, PAM8403 amplifier, I2S audio driver, Anycubic Cobra 3 Combo, 3D printing, Li-Po battery, smart home automation, Home Assistant speaker, DIY speaker system, DIY sound notifier, ESP32 projects, smart home DIY, Edifier R1380DB, compact speaker project, audio amplifier, tech projects, 3D printed case, IoT projects, home automation sound notifications

Уведомитель для Home Assistant

Представьте себе универсальное устройство-уведомитель, которое идеально впишется в вашу умную домашнюю систему. Оно позволяет получать звуковые и речевые уведомления, обеспечивает двустороннюю голосовую связь с Home Assistant и даже позволяет слушать музыку и радио — всё это без необходимости приобретать дополнительные Bluetooth-колонки. Будь то автоматизация повседневных задач или создание уютной атмосферы, такое устройство может вывести ваш умный дом на новый уровень.

Кроме того, этот уведомитель легко подключается к встроенным потолочным колонкам, что делает его незаметным, но мощным дополнением к любому помещению. Он упрощает интеграцию звука в умный дом, предоставляя функциональность и гибкость для самых разных сценариев использования.

С этими целями я решил создать свой собственный уведомитель. После двух предыдущих попыток, которые многому меня научили, я рад представить третью и наиболее доработанную версию уведомителя для Home Assistant. Вот как мне удалось её создать.

Модули печатных плат

Для этой версии уведомителя я использовал комбинацию компактных и эффективных модулей печатных плат, чтобы упростить конструкцию и улучшить функциональность:

- Плата Lolin32 Mini (или её аналоги)
Эта плата оказалась отличным выбором благодаря встроенному разъёму для аккумулятора Li-Po, зарядному устройству и защитным цепям. Эти функции позволили исключить необходимость в дополнительных модулях для управления батареей. Однако у неё отсутствует делитель напряжения для измерения заряда батареи, что является критически важной функцией для устройств на батарейном питании. Чтобы решить эту проблему, я припаял делитель напряжения между доступным GPIO, землёй и положительным контактом батареи.

- Стереодрайвер UDA1334 для вывода звука
Изначально я планировал использовать моноусилитель MAX98357, работающий по протоколу I2S, но такого модуля у меня не оказалось. Вместо этого я выбрал стереодрайвер UDA1334, который не имеет встроенных усилителей. Это оказалось преимуществом для моей конструкции, так как позволило подключить отдельный модуль усилителя, предоставив больше контроля над процессом усиления.

- Усилитель D-класса PAM8403
Для усиления я использовал компактный модуль PAM8403. Хотя это не самый идеальный выбор из-за ограничений в мощности и качестве звука, он был под рукой и вполне подошёл для нужд этого проекта. Несмотря на простоту, он отлично вписался в общую конструкцию, обеспечивая достаточную мощность для работы подключённой колонки.

Батарея и дизайн корпуса

В качестве источника питания я приобрёл в местном магазине аккумулятор Li-Po ёмкостью 1800 мА·ч. Он уже имел заранее обжатый разъём, совместимый с платой Lolin32 Mini, что значительно упростило процесс сборки.

Затем я приступил к разработке корпуса для будущего устройства. Если вам интересно, я могу подробно рассказать об этом процессе, но если это не в вашей зоне интересов, вы можете пропустить эту часть.

Когда дизайн корпуса был завершён, я отправил его на печать на моём недавно приобретённом 3D-принтере Anycubic Cobra 3 Combo. Принтер справился с задачей великолепно, обеспечив отличное качество и точность печати.

После печати я аккуратно разместил все компоненты внутри корпуса, закрепив их двусторонним силиконовым скотчем. Это обеспечило чистую и надёжную сборку без необходимости использования винтов или сложных креплений. В завершение я спаял все соединения, оживив устройство.

Тестирование и качество звука

Для тестирования устройства я подключил его к пассивной потолочной колонке, которая была у меня под рукой. Хотя качество звука этой базовой колонки не впечатляет, его оказалось более чем достаточно для выполнения основных задач — воспроизведения мягкой фоновой музыки и, что более важно, звуковых и речевых уведомлений от Home Assistant.

Для более тщательной проверки я подключил уведомитель к активным колонкам Edifier R1380DB, и результаты были потрясающими. Драйвер I2S на базе UDA1334 продемонстрировал удивительно высокое качество звука, учитывая его компактный размер и доступную цену. Чёткость и чистота звука превзошли мои ожидания, что делает его отличным выбором для использования с более качественными колонками.

Подведение итогов

На этом пока всё! Если вам понравился этот проект и вы хотите увидеть больше, не забудьте поставить лайк и подписаться на мой канал, чтобы не пропустить обновления о новых проектах. Делитесь своими мыслями и вопросами в комментариях — мне будет приятно услышать ваши отзывы и идеи!

Как всегда, вы можете найти список использованных деталей, схемы и 3D-модели для печати в описании к этому видео.

Спасибо за просмотр, и удачи в создании ваших собственных проектов! До встречи!